Tender No: Neurosurg./11-12/02

Technical Specification of C-Arm Image Intensifier

This equipment is used for imaging of bone pathology, fracture and displacement of bone with High Clarity.

A Versatile, Compact C arm Unit should allow unobstructed positioning and chanced case of operative intervention.

- Motorized vertical travel – 50 cm or more

- Pivotal rotation – 12.5⁰ or more

- Orbital rotation 90⁰ – 30⁰  or belter

- Radius of C-arm – adequate to encircle O.T. table and bulky patient

- Adequate free space between plate and x- ray tube – app. 75cm or more

- Rotation of C-arm – 80 cm or less

- Option of biplanar imaging

- Visualization of at least five vertebrae at a time

- C-arm should have facility of locking movement with easy to turn handle on control unit. Rear wheel must be freely movable for easy to control unit.

Image Intensifier – Minimum 9”

TV Camera –

Ultra compact CCD camera with high no of pixel and video band with two flicker free TV monitor with facility to rotate the image continuously.

Radiography should be possible on a cassette to be fitted in a holder for 10” x 12” cassette. The complete with one such folder and 1 no. cassettes including high speed intensifying screens.

X-Ray Generator -

Flinoscopy – output – 40 – 120 KV

MA Output – Minimum up to 8 mA or letter

Snapshot – Minimum up to 12.0 mA or better

Image Memory –

Adequate on hard disc should have facility to insert patient name through key board.

Unit must coated with dual channel laser light source on, X-ray inducing a cross to reduce X-ray disc, build in DODE area product meter for display to X-ray disc. Six lights weighted aprons required with DVD/CD writer.

Unit should be capable to function/store in ambient temperature of 0⁰-50⁰ and relative humidity of 20-90 %.

Power input to be 220-240 VAC, 50 Hz with Indian plug.

Over current circuit breaker / any other protection device.

Manufactures should have ISO certificate to quality standard. FDA approved certification preferred should fulfill the approved safety equipment of electrical equipment.

- Adequate staining to the staff

- User manual in English

- User list for 5 years is to be enclosed

- Faults have to be repaired within 48 hours at the site

- Equipment is to be provided at BHU

- Equipment should be of high Quality with excellent repute and Imported

- Validity not less than 31st March 2012